TRANSVERSE IMPEDANCE LIMITS

L. Vos

## I

# Transverse impedance of the LHC without collimators in 2003 (in M/m):

| injection |  | top energy |  |
| --- | --- | --- | --- |
| 8 kHz | 20 MHz | 8 kHz | 20 MHz |
| 56 | 10 | 110 | 10 |

- Transverse impedance of collimators, situation ~ 2001

octant 7	Al	3 x 0.2 m			octant 3	Al	1 x 0.2 m

Cu	12 x 0.5 m					Cu	4 x 0.5 m

| injection |  | top energy |  |
| --- | --- | --- | --- |
| 8 kHz | 20 MHz | 8 kHz | 20 MHz |
| 1 | 0.1 | 60 | 6 |

| injection |  | top energy |  |
| --- | --- | --- | --- |
| 8 kHz | 20 MHz | 8 kHz | 20 MHz |
| 56 | 10 | 110 | 10 |

| injection |  | top energy |  |
| --- | --- | --- | --- |
| 8 kHz | 20 MHz | 8 kHz | 20 MHz |
| 56 | 10 | 110 | 10 |

- Transverse impedance of graphite collimators 2003

( worst case )

| injection |  | top (collim. IN) |  |
| --- | --- | --- | --- |
| 8 kHz | 20 MHz | 8 kHz | 20 MHz |
| 20 | 14 | 1000 | 1040 |

- Total transverse impedance with graphite collimators

| injection |  | top (collim = injec) |  | top (collim. IN) |  |
| --- | --- | --- | --- | --- | --- |
| 8 kHz | 20 MHz | 8 kHz | 20 MHz | 8 kHz | 20 MHz |
| 76 | 24 | 130 | 24 | 1110 | 1050 |

COMMENTS :

The nearly constant impedance over the 20 MHz frequency band is a property of the objects with high resistance ( TDI, dump kicker, graphite collimators). It may cause problems with the transverse feedback system.

- Injection

The impedance reduction is only a factor of 3. However the decoherence time imposes a gain which is 4 times larger at low frequencies than required by stability alone. Hence a gain roll-off of a factor 12 (22 dB) between low frequency and 20 MHz is still acceptable here.

- Top energy squeezed

Problem : no roll-off allowed

Possible solution ? :  see later

Question :

what is the maximum gain that the feedback can offer at top energy at 20 MHz?

II

- Landau damping by beam-beam

The long range beam-beam appears during the low  squeeze.

The full beam-beam is active when full luminosity steering is achieved. It must ensure transverse stability since the feedback can not be kept on during collisions due to unavoidable emittance blow-up.

The results of the analysis of the stability diagrams based on the frequency distribution functions provided by W. Herr are :

(Assume an impedance dominated by the collimators)

- Long range alone

ZT < 90 M/m

largely insufficient

- Collisions

nominal intensity(3 IP) 		ZT < 1800 M/m

nominal intensity(1 IP) 		ZT < 450 M/m

ultimate intensity(2 IP) 		ZT < 1000 M/m

warning !

beware of larger than nominal emittance !

- Note on octupoles

The octupoles can make a tune shift at 1  of 10-4. They can stabilise 220 M/m at nominal intensity and 130 M/m at ultimate intensity (no margin taken).

tune /10-4

M/m

Long range

tune / 10-3

M/m

Collision nominal

##### Collision ultimate

tune / 10-3

M/m

Possible solution (?) for HF stability gap between squeeze and collisions

Impedance during unstable operational scenario :

|  | 8 kHz | 20 MHz | #### Landau damping | #### FB |
| --- | --- | --- | --- | --- |
| 7 TeV/c collim = injec | 130 | 24 | octupoles | off |
| 7 TeV/c collimators IN | 1110 | 1050 | insufficient | ? |
| squeeze | 1110 | 1050 | insufficient | ? |
| collisions | 1110 | 1050 | BB OK | off |

Impedance during stable operational scenario :

|  | 8 kHz | 20 MHz | #### Landau damping | #### FB |
| --- | --- | --- | --- | --- |
| 7 TeV/c collim = injec | 130 | 24 | octupoles | off |
| collisions | 130 | 24 | BB no octupoles | off |
| collisions collimators IN | 1110 | 1050 | BB OK | off |
| squeeze while keeping collisions | 1110 | 1050 | BB OK | off |

Solves stability gap and emittance blow-up during collision steering., but...

idea of squeezing the optics while keeping beams in collision not very popular

Identified limits :

[ultimate beam :  ZT < < 1000 M/m]

nominal  		ZT <  220 M/m  or  impedance defined by maximum gain feedback at 20 MHz, whichever is largest.

### III

- Emittance blow-up between squeeze and stable beams

Example for nominal beam intensity.

RMS tune spread in the beam from 4 10-5 (squeeze) to 5 10-4 (equivalent to 1100 M/m)

squeeze

growth time / sec

‘stable beam’

resolution 1 m

resolution 3 m